This file contains two datasheets, one for a jack-up, and the other for a semi submersible. The models are identical as used in various recent publications (see also the introduction document)

- STAB 2009, St Petersburg,

- ISSW, Brest 2013

- Jack-up conference in London, 2009

Note that stability analysis of offshore structures is different from ships. An important aspect is that the regulations require the direction of the heeling axis to be selected such that the “most critical” axis direction is found. For ships, the direction of the heeling axis is parallel to the longitudinal axis and is usually fixed. For offshore rigs the direction of the heeling axis can be any value. As touched upon in the above mentioned publications and also by various other authors, the selection of the heeling axis is not a simple task. Further reference is made to the papers and to the references in the papers.

The test cases presented here are meant for validation purposes only. They are made using GustoMSC’s program (DAMAST) which is used on numerous occasions for various offshore rigs. The models are quite simple which is done on purpose.

Reference: GustoMSC – Offshore Stability - Jack-up & Semi-sub data (Last update:12 February 2014)

| Structure information | 1 Jack-up: This jack-up was originally found in a publication of ABS and is purely a schematic shape not a copy of a real design. Reference: Breuer, J.A. and Sjöland, K.G., 2006, “Orthogonal Tipping in Conventional Offshore Stability Evaluations”, Proceedings of the 9th International conference on Stability of Ships and Ocean Vehicles (STAB 2006). 2 Semi submersible: This semi submersible is found in a publication of GustoMSC and is purely a schematic shape not a copy of a real design. Reference: Santen, JA van, The use of energy build up to identify the most critical heeling axis direction for stability calculations for floating offshore structures, STAB 2009 Conference St Petersburg |
| --- | --- |
| Hull geometry data | - Plan of form as found in the ABS and GustoMSC publications - 3D hull form (Rhino+IGES, kindly prepared by Gabriele Bulian) For the semi submersible take care to have a proper width for the damaged compartment of 6 m instead of 7 as found in an earlier version of the publication |
| Loading condition(s) | One draught, one KG intact and damaged |
| Numerical data | Calculation of the righting arms for various cases defined by axis direction and calculation method. |
| Enclosed documentation | - 1 Santen, J.A., The use of energy build up to identify the most critical heeling axis direction for stability calculations for floating offshore structures, review of various methods, 12th Jack Up Conference 2009, City University, London - Results with GustoMSC’s program DAMAST |
| Contact(s) for additional information/data | For any request of information please contact: JA van Santen: - e-mail: joost.vansanten@gustomsc.com - Phone: +31 10 2883 824 F Blaauw - e-mail: floris.blaauw@gustomsc.com - Phone: +31 10 2883 852 Common address: GustoMSC PO box 687 3100AR Schiedam The Netherlands |

Shapes

Jack-up

Semi submersible